# FSM State Diagram

## 1. Introduction

The Finite State Machine (FSM) controls the complete operation of the FPGA-Based Digital Voting Machine. It coordinates vote acceptance, vote storage, waiting for additional votes, and displaying the final result.

## 2. FSM State Diagram

## 3. FSM States

### IDLE

Initial state after reset. The system waits for a valid vote request.

### STORE_VOTE

Stores the selected candidate's vote and updates the vote counter.

### WAIT_VOTE

Waits for the next vote or the Show Result command.

### SHOW_RESULT

Determines the winning candidate and drives the 7-segment display.

## 4. State Transitions

The FSM transitions from IDLE to STORE_VOTE when a valid debounced vote is detected. After storing the vote, it moves to WAIT_VOTE. While in WAIT_VOTE, additional votes return the FSM to STORE_VOTE, whereas the Show Result command transfers the FSM to SHOW_RESULT. A system reset returns the FSM to the IDLE state.

## 5. Advantages of FSM Control

Provides deterministic system behavior.

Prevents invalid voting sequences.

Separates control logic from datapath modules.

Improves modularity and maintainability.

## 6. Conclusion

The FSM provides reliable control of the voting process by ensuring that votes are accepted, stored, and displayed in the correct sequence.