# State Encoding Table

## 1. Introduction

State encoding assigns a unique binary value to each state in the Finite State Machine (FSM). The Digital Voting Machine uses a 2-bit encoding because the controller consists of four states.

## 2. State Encoding Table

| State | Binary Code | Description |
| --- | --- | --- |
| IDLE | 00 | System waits for a valid vote. |
| STORE_VOTE | 01 | Stores the selected candidate's vote. |
| WAIT_VOTE | 10 | Waits for another vote or result request. |
| SHOW_RESULT | 11 | Displays the winning candidate. |

## 3. State Description

The IDLE state is entered after reset. When a valid debounced vote is detected, the FSM moves to STORE_VOTE to update the selected candidate's count. The WAIT_VOTE state allows additional votes to be accepted or waits for the Show Result command. Finally, the SHOW_RESULT state determines the winner and displays it on the 7-segment display.

## 4. Advantages of Binary State Encoding

Simple implementation in Verilog HDL.

Requires only two flip-flops for four states.

Efficient FPGA resource utilization.

Easy debugging during simulation.

## 5. Conclusion

The selected binary state encoding provides an efficient and reliable implementation of the voting controller while minimizing hardware resources.